New Stars Phoenix Coils

- Locations determined by matching wire colors from connector to coil

- Transistor numbers taken from Farfalla documentation

- Left PF Insert Flasher has not been determined

- Flashers under Spiral Ramp have not been determined

- Cabinet  C17-C  	Pins 1 – 4

- Playfield C17-P1 	Pins 5-16

- Playfield C17-P2	Pins 17-18

- Backbox C17-T	Pins 19,20

| Pin on C-17 | Wire Color | Location | Transistor |
| --- | --- | --- | --- |
| 1 | Red-White | Coin Door | Q20 |
| 2 | Pink-White | Knocker | Q15 |
| 3 | Pink-Yellow | Token Dispenser | Q10 |
| 4 | Purple-Yellow | Flasher – PF Backboard | Q5 |
| 5 | Yellow-White | Right Pop Bumper | Q19 |
| 6 | Brown-White | Outhole | Q24 |
| 7 | Blue-White | Right Sling | Q14 |
| 8 | Green-White | Left Sling | Q9 |
| 9 | Green-Purple | Ball Trough/ Shooter Lane | Q4 |
| 10 | Green-Pink | Saucer Eject / Spiral Ramp | Q18 |
| 11 | Pink-Yellow | Left Pop Bumper | Q23 |
| 12 | Pink-White | Kicker (Under Left Ramp) | Q13 |
| 13 | Yellow-Purple | Left Ramp Diverter | Q8 |
| 14 | Grey-White | Upper Left Flasher | Q3 |
| 15 | Grey-Black | Lower Flasher (Left & Right) | Q17 |
| 16 | Green-Black | Insert Flasher – Rt Ramp Entrance | Q22 |
| 17 | Yellow-Grey | Insert Flasher – Between Pops | Q12 |
| 18 | Pink-White | Flip Up Target Release | Q7 |
| 19 |  | Insert Flasher | Q2 |
| 20 |  | Insert Flasher | Q16 |
| 21 |  | Not Used | Q21 |
| 22 |  | Not Used | Q11 |
| 23 |  | Not Used | Q6 |
| 24 |  | Not Used | Q1 |