New Stars Phoenix Lamps

- Locations determined by grounding a leg on the SCR and noting which light came on.

- Connector/Pin information taken from Farfalla schematics

- CN22 – Back Box

- CN21 – Not Used

- CN20 – Playfield

- CN19 – Playfield

- CN18 – Playfield

- Back Box inserts not checked (anything CN22)

- Larger SCRs, 65 – 80 not tested yet

- There are 2 loose wires that I cannot identify their location

- SCRs on CN18-CN20 that did not trigger any lights, are marked UNK (unknown)

- Some SCRs had multiple outputs.  If one was on CN21 it is not noted here.

- The following light is not working in attract mode:

- Bonus – 5

- The following is a partial list of lights not located in this test

- Flip up inserts, numbered 1-5

- G in game

- One of the Red Special inserts (5 of 5, Top)

- B in ABCD

- 60x

- Bonus – 5 (not working in attract mode either)

| SCR Number | Location | Connector – Pin Number |
| --- | --- | --- |
| 1 |  | 22-18 |
| 2 |  | 22-17 |
| 3 |  | 22-16 |
| 4 | Bonus - 40x | 20-1 |
| 5 | Bonus – 3 | 19-16 |
| 6 | Shoot Again (?) | 19-10 |
| 7 |  | 22-3 |
| 8 |  | 22-4 |
| 9 |  | 22-5 |
| 10 |  | 22-14 |
| 11 |  | 22-13 |
| 12 |  | 22-12 |
| 13 | Bonus – 6 | 20-6 |
| 14 | UNK | 19-17 |
| 15 | Advance Red Special | 19-11 |
| 16 | 300,000 (under Advance Red Special) | 19-4 |
| 17 | 'E' in Game | 19-2 |
| 18 | Jumper Holes – Right | 18-18 |
| 19 |  | 22-9 |
| 20 | UNK | 20-15 |
| 21 | Bonus – 20x | 20-11 |
| 22 |  | 22-8 |
| 23 | Bonus – 1 | 19-18 |
| 24 | Bonus – 2 | 19-12 |
| 25 | UNK | 19-5 |
| 26 | C (in ABCD inserts) | 19-6 |
| 27 | Jumper Holes – Center | 18-17 |
| 28 | UNK | 20-20 & 22-2 |
| 29 | UNK | 20-19 |
| 30 | Red Special, bottom ( 1 of 5 ) | 20-10 |
| 31 | Bonus – 7 | 20-5 |
| 32 | Bonus – 9 | 19-20 |
| 33 | Red Special (4 of 5) | 19-13 |
| 34 | Red Special, middle (3 of 5) | 19-7 |
| 35 | 200,000 (Orange Special) | 18-20 |
| 36 | A in Game | 18-15 |
| 37 | Not Used | 21-17 |
| 38 | UNK | 20-14 |
| 39 | UNK | 20-9 |
| 40 | Bonus – 80x | 20-2 |
| 41 | Not Used | 21-16 |
| 42 | Not Used | 21-15 |
| 43 | Not Used | 21-14 |
| 44 | 50,000 (Orange Special) | 18-19 |
| 45 | 300,000 (Orange Special) | 18-4 |
| 46 | A in ABCD | 20-17 |
| 47 | UNK | 20-13 |
| 48 | UNK | 20-8 |
| 49 | Bonus – 10 | 20-4 |
| 50 | Not Used | 21-12 |
| 51 | Bonus – 4 | 19-15 |
| 52 | Red Special (2 of 5) | 19-7 |
| 53 | Jumper Holes – Left | 21-10 |
| 54 | 100,000 (Orange Special) | 18-13 |
| 55 | D in ABCD | 20-16 |
| 56 | UNK | 20-12 |
| 57 | UNK | 20-7 |
| 58 | 10,000 (By Bonus X) | 20-3 |
| 59 | UNK | 19-3 |
| 60 | Bonus – 8 | 19-14 |
| 61 | Advance Orange Special | 19-8 |
| 62 | M in Game | 19-3 |
| 63 | UNK | 18-12 |
| 64 | Flipper Relay | 16-4 |
| 65 |  | 22-10 |
| 66 |  | 18-9 |
| 67 |  | 18-8 |
| 68 |  | 22-15 |
| 69 |  | 18-7 |
| 70 |  | 18-10 |
| 71 |  | 18-11 |
| 72 |  | 18-6 |
| 73 |  | 18-5 |
| 74 |  | 22-6 |
| 75 |  | 18-4 |
| 76 |  | 21-18 |
| 77 |  | 18-3 |
| 78 |  | 18-1 |
| 79 |  | 18-22 |
| 80 |  | 18-16 |